/****************************************************************************/

/*D

netgen - 3d grid generator

INSTALLATION:

Do the following steps.

.n 1) Fill out the netgenlicence $(UGROOT)/gm/gg3/netgenlisencs.ps.

.n 2) You will obtain the missing file ruler3.cc.

.n    Copy it in $(UGROOT)/gm/gg3/netgen/meshing.

.n 3) Set NETGEN = ON in ug.conf.

.n 4) Edit the machines file of your mk.arch.

.n    Fill in ARCH_C++ and ARCH_C++FLAGS

.n 5) Call 'ugmake netgen'.

HOW TO USE THE GRIDGENERATOR:

Include

.vb

bnodes $h 0.5;

makegrid $s 2;

.ve

in your script. This generates a surface triangulation with step size 0.5

and it generates a grid. Then, 2 smoothing steps are performed to

optimize the inner angles. See $(UGROOT)/../cd/appl/scripts/mg.init for an example.

SHORT DESCRIPTION:

In this directory and below you find a part of the 3D mesh-generator NETGEN,

developed by Joachim Schoeberl at the Johannes Kepler Universitaet Linz.

This part is able to dissect a volume given by boundary triangles into

tetrahedrons. Also a mesh smoother is contained in this package.

The source files are splitted into the following directories.

. meshing -     files for mesh-generation

. general -     files for abstract data types (ARRAY, HASHTABLE, ...)

. geom    -     files for geometric objects

. linalg  -     files for linear algebra

. opti    -     files for BFGS - optimization

The principle of NETGEN is explaind in

Joachim Schoeber,

An Automatic Mesh Generator using Geometric Rules

for two and three Space Dimensions

Techn. Rep. No 95-3, JKU-Linz

This paper is available via internet. Please have a look into

www.numa.uni-linz.ac.at

D*/

/****************************************************************************/